ECE 1212 - Electronic Circuit Design Laboratory

Pre-Lab 1A

Question 1(a) - Non-Inverting Amplifier, Av=1

The minimum-component design is a voltage follower. The output is connected directly to the inverting input, and the signal is applied to the non-inverting input.

Draw A_v = 1 schematic here

Gain and bandwidth prediction

Av = 1

For a single-pole op-amp, approximately:  ft=Av*f3dB

Therefore:  f3dB=ft/Av=ft

Using the nominal 741 unity-gain bandwidth of about 1 MHz as a pre-lab estimate:

Predicted f3dB =1MHz

Application of the Av=1 amplifier

The unity-gain amplifier is a voltage follower or buffer. It does not increase the signal voltage, but it provides a very high input impedance and a low output impedance. It is useful for isolating two circuit stages so that a load does not significantly disturb the signal source, and for improving the ability of a weak source to drive a following stage.

Pre-Lab 1A

Question 1(b) - Non-Inverting Amplifier, Av=10

For a non-inverting amplifier:

Av=1+Rf/Rg

Set Av=10:

10=1+Rf/Rg->Rf/Rg=9

Choose: Rg=1.0kΩ,Rf=9.0kΩ

Draw A_v = 10 schematic here

Bandwidth prediction

f3dB =ft/Av

Using ft =1 MHz for the pre-lab estimate:

f3dB =1MHz/10=100kHz

Pre-Lab 1A

Question 2 - Input Impedance of the Non-Inverting Amplifier at 1 kHz

Draw input-impedance measurement circuit here

Testing procedure

1. Set the signal generator to a 1 kHz sine wave with zero DC offset.

2. Insert a known series resistor Rs between the signal generator and the non-inverting input of the amplifier.

3. Measure the source-side voltage Vs and the amplifier input voltage Vin with the oscilloscope.

4. Treat R_s and Z_in as a voltage divider and calculate the input impedance from the measured voltages.

Calculation

Vin=Vs*Zin/Rs+Zin

Therefore:  Zin=Rs*Vin/Vs-Vin

If Vin=Vs/2, then Zin=Rs.

Predicted input impedance

For the ideal op-amp model, the input current is zero, so the input impedance of a non-inverting amplifier is infinite. For a real 741, it is finite but very high. Negative feedback increases the effective input impedance, so both Av=1 and Av=10 should have a high input impedance; the unity-gain follower is expected to have the larger effective input impedance because its feedback factor is larger.

Pre-Lab 1B

Question 1 - Inverting Amplifier Design

Draw inverting-amplifier schematic here

Step 1 - Choose the input resistor

For an inverting amplifier at low frequency:  Zin =Rin

Required Zin=1kΩ->choose Rin=1.0kΩ

Step 2 - Use the measured unity-gain bandwidth

Provisional measured value selected from Part I: ft=300kHz

This is obtained from the Av = 10 measurement:  ft =10*30kHz=300kHz.

Step 3 - Find the largest feedback resistor that still gives f_3dB >= 20 kHz

For an inverting amplifier, the bandwidth is set by the noise gain:

Noise gain = 1+Rf/Rin ,f3dB =ft/1+Rf/Rin

Require:  20kHz<=300kHz/1+Rf/1kΩ,1+Rf/1kΩ<=15,Rf/1kΩ<=14->Rf<=14kΩ

Choose Rf=14kΩ for the theoretical maximum based on this provisional f_t.

Step 4 - Gain and bandwidth

Signal gain: Av=-Rf/Rin=-14kΩ/1kΩ=-14

Predicted f3dB =300kHz/1+14=20kHz

Load: RL=3.3kΩ connected from output to ground.

Pre-Lab 1B

Question 2 - DC Compensation Modification

Draw modified DC-compensation schematic here

How the DC compensation works

Add an adjustable DC compensation voltage to the inverting summing node through a resistor. The potentiometer (or decade resistor arrangement) is adjusted so that the DC contribution from the compensation path cancels the DC component of the input signal. The AC component then remains and is amplified normally.

For a two-input inverting summer:

Vout=-Rf*Vin/Rin+Vcomp/Rc

For the DC component, set Vout, DC = 0:

VDC/Rin+Vcomp/Rc=0

Therefore: Vcomp=-VDC*Rc/Rin

A convenient simple choice is Rc=Rin,  giving Vcomp=-VDC.

Pre-Lab 1B

Question 3 - Input Impedance of the Inverting Amplifier

Draw input-impedance measurement circuit here

Testing procedure

1. Insert a known series resistor R_s between the signal generator and the amplifier input.

2. Measure the source-side voltage V_s and the voltage V_in at the amplifier input.

3. Calculate the input impedance from the voltage-divider relation.

4. Repeat the measurement at 100 Hz, 1 kHz, 5 kHz, 15 kHz, and several higher frequencies.

Calculation

Zin=Rs*Vin/Vs-Vin

Low-frequency prediction

At low frequency, negative feedback keeps the inverting input close to virtual ground, so the source sees approximately the input resistor:

Zin =Rin=1.0kΩ

Expected variation with frequency

As frequency increases, the 741 open-loop gain decreases and the virtual-ground approximation becomes less exact. Therefore the measured input impedance will gradually deviate from the low-frequency value of about 1 kOhm. The exact frequency-dependent equation should be taken from the course lecture notes, as requested by the handout.

| Frequency | Pre-lab prediction |
| --- | --- |
| 100 Hz | Approximately 1.0 kOhm |
| 1 kHz | Approximately 1.0 kOhm |
| 5 kHz | Approximately 1.0 kOhm; small deviation possible |
| 15 kHz | May begin to deviate from 1.0 kOhm |
| Higher frequencies | Increasing deviation as loop gain falls |